NILEGATE-DOC v1

document_code: NG-OPS-PRC-0042

title: Reefer Container Pre-Trip Inspection Procedure

document_type: procedure

department: Operations

sensitivity_level: 1

version_date: 2026-03-15

language: en

data_origin: internal

personal_data: no

END-METADATA

# 1. Purpose

This procedure sets out how NileGate terminal technicians inspect refrigerated (reefer) containers before they are released for loading or handed over to a shipper, so that temperature-sensitive cargo is never stowed in a unit that cannot hold its set point.

# 2. Scope

It applies to every 20-foot and 40-foot reefer unit handled at NileGate terminals, including shipper-owned units. Diesel generator sets clipped to reefers are inspected under a separate procedure. The procedure binds reefer technicians, yard planners and shift supervisors.

# 3. Responsibilities

## 3.1 Reefer technician

The reefer technician performs the pre-trip inspection (PTI) and records the result in the terminal operating system within 30 minutes of finishing it. Every inspected unit receives a green PTI PASS tag or a red PTI FAIL tag on the left-hand door.

## 3.2 Yard planner

The yard planner allocates reefer slots that have a working power point and never plans a unit with a failed or missing PTI for loading.

## 3.3 Shift supervisor

The shift supervisor signs off the daily PTI log at the end of each shift and escalates any unit that fails twice to the equipment maintenance department.

# 4. Pre-trip inspection steps

## 4.1 Visual check

The technician checks the door gaskets, the drain plugs, the condenser coil and the control panel. Any hole or tear in the walls or the floor larger than 5 mm, a missing drain plug or a cracked panel display fails the unit at once.

## 4.2 Electrical check

The plug and the power cable are inspected for cuts and burnt pins before the unit is connected to a 440 V power point. The insulation resistance of the compressor circuit must be at least 1 megaohm; a lower reading fails the unit.

## 4.3 Functional test

The technician runs the controller's automatic PTI, then a pull-down test to the cargo set point. A 40-foot unit must reach minus 18 degrees Celsius within 90 minutes and a 20-foot unit within 60 minutes; the data logger must record the whole test.

# 5. Temperature set points and tolerances

Frozen cargo is carried at minus 18 degrees Celsius or colder with a tolerance of plus or minus 1 degree. Chilled cargo is carried between 0 and 13 degrees Celsius with a tolerance of plus or minus 0.5 degree. Set points are always entered in Celsius, never in Fahrenheit.

| Cargo | Set point | Tolerance |
| --- | --- | --- |
| Frozen meat and fish | -18 °C or colder | ±1 °C |
| Chilled fruit | 0 to +13 °C | ±0.5 °C |
| Bananas | +13.3 °C | ±0.5 °C |

# 6. Failed units

A failed unit is tagged PTI FAIL and moved to the repair area within 4 hours. The shipping line is notified within 2 hours through the equipment status message. A unit that fails its PTI twice in 30 days is withdrawn from service for a full overhaul.

# 7. Records

PTI records, including the downloaded data logger file, are kept for 2 years and are shown to the shipping line on request.